import pandas as pd

import numpy as np

import glob

import os

folder_path = r"C:\Users\nashd\OneDrive\Desktop\AUTOTWIN_Dashboard\B0047-Charge"

all_files = glob.glob(folder_path + "/*.csv")

results = []

for file in all_files:

df = pd.read_csv(file)

# Convert Time column to seconds

df['Time'] = pd.to_datetime(df['Time'])

df['DeltaT'] = df['Time'].diff().dt.total_seconds()

# Replace NaN for first row

df['DeltaT'] = df['DeltaT'].fillna(0)

# Capacity calculation

# Current is in Amps, time in seconds → convert to Ah

df['CapacityContribution'] = df['Current_measured'] * df['DeltaT'] / 3600

cycle_capacity = df['CapacityContribution'].sum()

results.append({

"File": os.path.basename(file),

"AvgVoltage": df['Voltage_measured'].mean(),

"AvgCurrent": df['Current_measured'].mean(),

"MaxTemp": df['Temperature_measured'].max(),

"CycleCapacity_Ah": cycle_capacity

})

# Final table

results_df = pd.DataFrame(results)

print(results_df)