"""

thevenin_ecm.py  —  AUTOTWIN | Thevenin 1RC Equivalent Circuit Model

======================================================================

Backend physics engine for NASA battery discharge data.

Model Topology (1RC Thevenin):

┌──── R0 ────┬──── R1 ────┐

│            │            │

OCV(SOC)     C1          V_terminal

│            │            │

└────────────┴────────────┘

Equations

---------

V_t[k]    = OCV(SOC[k]) + I[k]*R0 + V_RC[k]

V_RC[k+1] = V_RC[k]*exp(-dt/tau) + I[k]*R1*(1 - exp(-dt/tau))

SOC[k]    = SOC[0] - integral(|I|dt) / Q_nominal

Parameter Identification: Two-stage — global (Differential Evolution)

followed by local (L-BFGS-B) optimisation.

Usage

-----

from thevenin_ecm import TheveninECM

ecm = TheveninECM()

results = ecm.run(df, Q_nominal_Ah=2.0)

"""

import numpy as np

import pandas as pd

from scipy.optimize import differential_evolution, minimize

from scipy.integrate import cumulative_trapezoid

# ─────────────────────────────────────────────────────────────────────────────

#  CONSTANTS

# ─────────────────────────────────────────────────────────────────────────────

NASA_Q_NOMINAL = 2.0   # Rated capacity for fresh NASA 18650 cells (Ah)

# OCV-SOC look-up table (18650 NMC, calibrated to NASA B00xx family)

_SOC_LUT = np.linspace(0.0, 1.0, 21)

_OCV_LUT = np.array([

2.70, 2.90, 3.10, 3.22, 3.32, 3.40, 3.46, 3.51, 3.56, 3.60, 3.65,

3.69, 3.73, 3.77, 3.82, 3.87, 3.93, 3.99, 4.05, 4.13, 4.20

])

_OCV_POLY_DEGREE = 8

# ─────────────────────────────────────────────────────────────────────────────

#  MAIN CLASS

# ─────────────────────────────────────────────────────────────────────────────

class TheveninECM:

"""

1RC Thevenin Equivalent Circuit Model for Li-ion batteries.

Self-calibrates the OCV-SOC polynomial to each battery file and uses

a two-stage global + local optimiser to identify R0, R1, C1.

"""

_BOUNDS = [

(0.001, 0.50),      # R0 (Ohm)

(0.001, 0.50),      # R1 (Ohm)

(50.0,  20000.0),   # C1 (F)

]

def __init__(self):

self.R0  = None

self.R1  = None

self.C1  = None

self.tau = None

self._ocv_poly = np.polyfit(_SOC_LUT, _OCV_LUT, _OCV_POLY_DEGREE)

self._fitted = False

# ── Public API ────────────────────────────────────────────────────────────

def run(self, df, Q_nominal_Ah=NASA_Q_NOMINAL, verbose=False):

"""

Full pipeline: preprocess -> SOC -> calibrate OCV ->

identify params -> simulate -> metrics.

Parameters

----------

df : pd.DataFrame   NASA discharge CSV as a DataFrame

Q_nominal_Ah : float   Cell rated capacity in Ah

verbose : bool          Print optimiser progress

Returns

-------

dict with keys:

params, metrics, time, V_measured, V_simulated, soc, current,

temperature (if column exists in df)

"""

df = self._preprocess(df)

if df is None or len(df) < 10:

raise ValueError("Too few discharge samples after preprocessing.")

soc = self._coulomb_count(df, Q_nominal_Ah)

self._calibrate_ocv(df, soc)

self._identify_parameters(df, soc, verbose)

V_sim = self._simulate(

df["Time"].values,

df["Current_measured"].values,

soc, self.R0, self.R1, self.C1

)

metrics = self._compute_metrics(df["Voltage_measured"].values, V_sim)

result = {

"params": {

"R0_ohm": round(self.R0, 6),

"R1_ohm": round(self.R1, 6),

"C1_F":   round(self.C1, 4),

"tau_s":  round(self.tau, 4),

},

"metrics":     metrics,

"time":        df["Time"].values,

"V_measured":  df["Voltage_measured"].values,

"V_simulated": V_sim,

"soc":         soc,

"current":     df["Current_measured"].values,

"Q_nominal_Ah": Q_nominal_Ah,

}

if "Temperature_measured" in df.columns:

result["temperature"] = df["Temperature_measured"].values

return result

@staticmethod

def load_csv(filepath):

"""Load a NASA battery discharge CSV from a file path."""

df = pd.read_csv(filepath)

df.columns = df.columns.str.strip()

return df

@staticmethod

def load_uploaded(file_obj):

"""

Load a CSV from a Streamlit UploadedFile or file-like object.

"""

import io

raw = file_obj.read() if hasattr(file_obj, "read") else file_obj

df = pd.read_csv(io.BytesIO(raw))

df.columns = df.columns.str.strip()

return df

def ocv(self, soc):

"""Return OCV (V) for given SOC array using the calibrated polynomial."""

return np.polyval(self._ocv_poly, np.clip(soc, 0.0, 1.0))

# ── Internals ─────────────────────────────────────────────────────────────

def _preprocess(self, df):

required = {"Voltage_measured", "Current_measured", "Time"}

missing = required - set(df.columns)

if missing:

raise ValueError(f"CSV missing required columns: {missing}")

df = df.copy()

df = df.sort_values("Time").reset_index(drop=True)

# Keep only the discharge phase (current clearly negative)

df = df[df["Current_measured"] < -0.1].reset_index(drop=True)

if len(df) < 10:

return None

df["dt"] = df["Time"].diff().fillna(0).clip(lower=0)

return df

def _coulomb_count(self, df, Q_nominal_Ah):

"""Coulomb counting: SOC in [0,1], starts at 1.0."""

Q_As = Q_nominal_Ah * 3600.0

charge_out = cumulative_trapezoid(

np.abs(df["Current_measured"].values),

df["Time"].values,

initial=0

)

return np.clip(1.0 - charge_out / Q_As, 0.0, 1.0)

def _calibrate_ocv(self, df, soc):

"""

Adapt OCV-SOC polynomial to THIS battery using a coarse IR-corrected

voltage estimate: OCV_approx = V_measured - I*R0_coarse

"""

I = df["Current_measured"].values

V = df["Voltage_measured"].values

dV = np.abs(np.diff(V[:6]))

dI = np.abs(np.diff(I[:6]))

mask = dI > 0.05

R0_coarse = float(np.clip(

np.median(dV[mask] / dI[mask]) if mask.any() else 0.10,

0.01, 0.30

))

V_ocv_approx = V - I * R0_coarse

with np.errstate(all="ignore"):

self._ocv_poly = np.polyfit(soc, V_ocv_approx, _OCV_POLY_DEGREE)

def _simulate(self, time, current, soc, R0, R1, C1):

"""

Discrete-time 1RC Thevenin simulation using zero-order hold (ZOH).

alpha = exp(-dt / tau)

V_RC[k+1] = alpha*V_RC[k] + R1*(1-alpha)*I[k]

V_t[k]    = OCV(SOC[k]) + R0*I[k] + V_RC[k]

"""

n   = len(time)

tau = R1 * C1

V_RC = np.zeros(n)

V_t  = np.zeros(n)

V_t[0] = self.ocv(soc[0:1])[0] + current[0] * R0

for k in range(1, n):

dt    = max(float(time[k] - time[k-1]), 1e-6)

alpha = np.exp(-dt / tau) if tau > 1e-9 else 0.0

V_RC[k] = V_RC[k-1] * alpha + current[k-1] * R1 * (1.0 - alpha)

V_t[k]  = self.ocv(soc[k:k+1])[0] + current[k] * R0 + V_RC[k]

return V_t

def _identify_parameters(self, df, soc, verbose):

"""

Two-stage optimisation to minimise RMSE(V_measured, V_simulated).

Stage 1 — Differential Evolution (global, seed=42 for reproducibility)

Stage 2 — L-BFGS-B (local refinement from Stage 1 best)

"""

time    = df["Time"].values

current = df["Current_measured"].values

V_meas  = df["Voltage_measured"].values

def cost(x):

V_sim = self._simulate(time, current, soc, *x)

return np.sqrt(np.mean((V_sim - V_meas) ** 2))

if verbose:

print("[ECM] Stage 1 — Differential Evolution …")

de = differential_evolution(

cost, self._BOUNDS,

seed=42, maxiter=500, tol=1e-7,

popsize=15, mutation=(0.5, 1.5), recombination=0.75,

workers=1, polish=False

)

if verbose:

print(f"[ECM] Stage 1 RMSE = {de.fun*1000:.3f} mV")

print("[ECM] Stage 2 — L-BFGS-B refinement …")

local = minimize(

cost, de.x, method="L-BFGS-B", bounds=self._BOUNDS,

options={"maxiter": 3000, "ftol": 1e-13, "gtol": 1e-11}

)

if verbose:

print(f"[ECM] Stage 2 RMSE = {local.fun*1000:.3f} mV")

self.R0  = float(local.x[0])

self.R1  = float(local.x[1])

self.C1  = float(local.x[2])

self.tau = self.R1 * self.C1

self._fitted = True

@staticmethod

def _compute_metrics(V_meas, V_sim):

err    = V_meas - V_sim

rmse   = float(np.sqrt(np.mean(err ** 2)))

mae    = float(np.mean(np.abs(err)))

ss_res = float(np.sum(err ** 2))

ss_tot = float(np.sum((V_meas - V_meas.mean()) ** 2))

r2     = 1.0 - ss_res / ss_tot if ss_tot > 0 else float("nan")

maxe   = float(np.max(np.abs(err)))

mape   = float(np.mean(np.abs(err / np.where(V_meas != 0, V_meas, 1e-9))) * 100)

return {

"RMSE_V":   round(rmse, 6),

"MAE_V":    round(mae,  6),

"R2":       round(r2,   6),

"MaxErr_V": round(maxe, 6),

"MAPE_pct": round(mape, 4),

}

# ─────────────────────────────────────────────────────────────────────────────

#  CLI ENTRY POINT

# ─────────────────────────────────────────────────────────────────────────────

if __name__ == "__main__":

import argparse, os, sys

parser = argparse.ArgumentParser(description="AUTOTWIN — Thevenin 1RC ECM")

parser.add_argument("--file",   required=True, help="Path to discharge CSV")

parser.add_argument("--qnom",   type=float, default=NASA_Q_NOMINAL)

parser.add_argument("--outdir", default=".", help="Output directory")

args = parser.parse_args()

if not os.path.isfile(args.file):

print(f"[ERROR] File not found: {args.file}", file=sys.stderr)

sys.exit(1)

os.makedirs(args.outdir, exist_ok=True)

print(f"\n{'='*55}\n  AUTOTWIN — Thevenin 1RC ECM\n  File: {args.file}\n{'='*55}")

ecm = TheveninECM()

raw = TheveninECM.load_csv(args.file)

res = ecm.run(raw, Q_nominal_Ah=args.qnom, verbose=True)

print("\n── Parameters ──────────────────────────────────")

for k, v in res["params"].items():

print(f"  {k:10s} : {v}")

print("\n── Metrics ─────────────────────────────────────")

for k, v in res["metrics"].items():

print(f"  {k:12s} : {v}")

soc = res["soc"]

print(f"\n── SOC ─────────────────────────────────────────")

print(f"  Initial : {soc[0]*100:.1f}%")

print(f"  Final   : {soc[-1]*100:.1f}%")

print(f"  DoD     : {(soc[0]-soc[-1])*100:.1f}%")

base    = os.path.splitext(os.path.basename(args.file))[0]

out_csv = os.path.join(args.outdir, f"{base}_ecm_output.csv")

pd.DataFrame({

"Time":        res["time"],

"V_measured":  res["V_measured"],

"V_simulated": res["V_simulated"],

"V_error_mV":  (res["V_measured"] - res["V_simulated"]) * 1000,

"SOC":         soc,

"Current":     res["current"],

}).to_csv(out_csv, index=False)

print(f"\n[OK] Results -> {out_csv}\n")